# EEG Movement Detection Pipeline Report

This report gives a simple explanation of the signal‑processing pipeline used to detect movement‑related activity from a multi‑channel EEG.

Paprapered By: Ahmet AJAM

The pipeline has four main ideas:

- Preprocess the EEG to remove drift and noises.

- Estimate the rest/baseline level of each channel.

- Measure activity inside short time‑windows using RMS.

- Detect movement when many channels show increased activity at the same time.

## 1. Time Handling & Sampling Rate Estimation

Let the recorded time column be denoted as (t_{raw}(n)). In some files the time values are expressed in milliseconds:

traw(n)=0,1,2,…,N,(ms).

We detect this case automatically and convert to seconds:

t(n)=traw(n)1000 [s].

The sampling period and sampling rate are then estimated as:

Δt=mediant(n+1)-t(n),  fs=1Δt.

For this dataset, typically (f_s = 1000 , {Hz}.)

## 2. Preprocessing: High‑Pass Filtering and Absolute Activation Extraction

The raw multi‑channel signal (x_i(n)) (channel index (i = 1,\dots,C)) is passed through a 2nd‑order Butterworth high‑pass filter to remove drift and slow baseline wandering:

xihp(n)=HPF0.5,Hzxi(n).

Since the signals are already slow and do not contain classical EMG burst frequencies, rectification is sufficient to obtain an activation‑intensity trace:

ai(n)=|xihp(n)|.

This activation signal (a_i(n)) is the primary amplitude domain used in subsequent steps.

## 3. Baseline Modeling from Rest Segment

A rest interval ([t_{r,0}, t_{r,1}]) is specified (e.g., the first 3 seconds). For each channel, samples in this interval are used to estimate baseline statistics:

airest=ai(n):t(n)∈[tr,0,tr,1].

We compute the median and median absolute deviation (MAD):

mi=median(airest),

MADi=median|airest-mi|.

These statistics model the noise floor for each channel.

### Dead‑Channel Detection

A channel is declared inactive and ignored if either:

- Its variance is extremely small, or

- (\mathrm{MAD}_i) is too close to zero.

Such channels contribute no useful activation information.

## 4. Normalization and Gating of Near‑Zero Activity

A robust z‑score representation is computed for visualization and gating:

zi(n)=ai(n)-miMADi.

To eliminate spurious micro‑fluctuations around baseline, a small gating threshold (\varepsilon) is applied:

ziclean(n)=0,|zi(n)|<ε, zi(n),otherwise.

Gate (\varepsilon) is typically in the range 0.2–0.3. Importantly, this gating is applied only to the normalized z‑representation; the true activation values (a_i(n)) remain unchanged for feature extraction.

## 5. Windowed Feature Extraction (RMS per Channel)

The activation signal is divided into overlapping windows of length (L_w) with hop size (H_w):

Lw=round(0.200,fs),  Hw=round(0.100,fs).

For each window (w) and channel (i), the root‑mean‑square (RMS) feature is computed:

RMS*i(w)=1Lw∑*n∈wai(n)2.

RMS captures the average activation intensity in that window.

The window center time is:

tw=t(nstart)+t(nend)2.

## 6. Per‑Channel Activation Detection & Global Consensus

### 6.1 Per‑Channel Thresholds

For each channel, RMS values from the rest windows are used to form a threshold:

θi=minmedian(RMSirest)*k,MAD(RMS*irest),;percentile*p(RMSirest).

Typical values:

- (k = 2.0)

- (p = 85% ).

### 6.2 Per‑Channel Activity Mask

A binary activity measure is computed per channel per window:

Ai(w)=1,RMSi(w)>θi, 0,otherwise.

### 6.3 Global Movement Consensus

Let (C_{alive}) be the number of non‑dead channels. The fraction of active channels is:

F(w)=1Calivei=1CaliveAi(w).

A window is classified as globally active when:

F(w)≥Fon.

Typical values:

- (F_{on} = 0.5)

- (F_{off} = 0.3).

## 7. Pulse Detection and Movement Segmentation

Global activity (F(w)) is processed using a hysteresis detector:

- A pulse starts when (F(w) \geq F_{on}).

- It ends when (F(w) \leq F_{off}).

Minimum duration and merging rules ensure physiologically meaningful intervals:

- Minimum pulse duration:

Tmin=150,ms.

- Merge pulses separated by less than (150 {ms}).

### Pulse Metrics

For each pulse, we compute:

- Start time: (t_{start})

- End time: (t_{end})

- Duration: (t_{end} - t_{start})

- Peak of (F(w)) and its time

- Rise time (T_{10 to 90})

- Fall time (T_{90 to 10})

- Area under curve (AUC):

AUC=tstarttendF(t),dt.